Assessment of irradiation modules diagnostics and capsule filling in IFMIF-DONES

Irene Álvarez1, Miguel Macías1, Juan A. De la Torre2, Gonzalo M. Martín1, Marta Anguiano3, Alberto Artacho1, Santiago Becerril1, Luis Monasterio1, Fernando Mota4, Iván Podadera1, Claudio Torregrosa-Martín1, Moisés Weber1

1Consorcio IFMIF-DONES España, Granada, Spain

2Universidad Autónoma de Barcelona, Barcelona, Spain

3Universidad de Granada, Granada, Spain

4CIEMAT, Madrid, Spain

Corresponding Author Email: irene.alvarez@ifmif-dones.es

The materials proposed for nuclear fusion applications have yet to be fully characterized under the neutron irradiation conditions expected in future fusion reactors. IFMIF-DONES (International Fusion Materials Irradiation Facility – DEMO Oriented NEutron Source) has been conceived to address this challenge by irradiating materials under neutron conditions representative of those found in fusion environments. In IFMIF-DONES, neutrons will be produced through the interaction of a 40 MeV, 125 mA deuteron beam, with a thick liquid lithium target, where deuteron–lithium reactions generate a high-intensity neutron flux.

Downstream of the lithium target, different modules will be placed for the commissioning and operation phase. These modules will host diagnostics to machine protection, beam monitoring and beam characterization. Among the diagnostics currently considered are ionization chambers, fission chambers, and self-powered neutron detectors. To better understand the behaviour of these detectors in the IFMIF-DONES environment, theoretical sensitivity studies have been carried out. These studies aim to gain deeper insight into the influence of the detector design on the estimation of the induced current.

Moreover, to mitigate temperature gradients induced by gaps between specimens, the irradiation module capsules and diagnostics will be filled with a conductive material to ensure thermal homogenization. At present, sodium has been selected for this purpose, as it behaves as a liquid metal at irradiation temperature. However, sodium is corrosive to the specimen materials, which motivates the need to investigate, from the neutronic point of view, alternative filling materials in solid, liquid and gas states that can provide adequate thermal performance while ensuring material compatibility and without jeopardizing the neutron doses received by the specimens.

The assessment of the different designs and scenarios was carried out using calculations obtained from different Monte Carlo simulation codes.

| Type of contribution: | Poster session ☐ |
| --- | --- |
|  | Oral Contribution ☒ |